MAC Flooding Attack

- Attack

  - dsniff on kali

  - macof -n 5

- Defence (port security)

  - Dynamic:

    - switchport port-security

  - Maximum:

    - switchport port-security

    - switchport port-security maximum 8

  - Sticky

    - switchport port-security

    - switchport port-security mac-address sticky

  - Static

    - switchport port-security

    - switchport port-security mac-address 0000.0000.0000

  - Recovery

    - errdisable recovery cause ?

Switch Spoofing Attack

- Attack

  - Yersinia on kali

- Defence

  - Disable DTP negotiations and manually configure ports as access or trunk explicitly

    - switchport mode access

    - switchport access vlan 10

    - switchport nonegotiate

  - shutdown all unused ports and place them in an unused VLAN

    - switchport mode access

    - switchport access vlan 100 (unused vlan)

    - shutdown

    - switchport trunk allowed vlan remove 100 (remove unused vlan)

Double-Tagging Attack

- Attack

  - Yersinia on Kali

- Defence

  - configure dedicated native VLAN, e.g., VLAN 500, for all trunk ports which should not be assigned to any access ports.

    - switchport trunk native vlan 500

    - vlan dot1q tag native

STP Attack

- Attack

  - Use a rogue cisco switch and plug it in (Lab 2, pg 16 notes)

- Defence

  - apply BPDU guard feature available

    - spanning-tree bpduguard enable

  - BPDU guard may simply be enabled globally on all PortFast-enabled ports since these ports should not receive BPDU.

    - spanning-tree portfast

    - spanning-tree portfast bpduguard default

  - root guard feature to enforce the placement of root switches, preventing them from being taken over by rogue root switch.

    - spanning-tree guard root

CDP Information Leakage

- Attack

  - Capture with wireshark

- Defence

  - disable CDP individually on all access ports, except ports supporting VoIP which require CDP to function.

    - no cdp enable

  - disabled globally on all interfaces of the device

    - no cdp run

DHCP Starvation and Spoofing Attack that may lead to DNS and website spoofing

- Attack

  - Yersinia on Kali (DHCP Starvation Attack)

  - Metasploit on Kali (DHCP Server Spoofing Attack)

  - XAMPP on Windows (Spoof Website Attack)

  - DNSChef on Kali (DNS Server Spoofing Attack)

- Defence

  - DHCP Snooping

    - ip dhcp snooping

    - ip dhcp snooping vlan vlan-id

    - ip dhcp snooping verify mac-address (DHCP snooping MAC address verification)

    - ip dhcp snooping limit rate rate (DHCP snooping rate-limiting (from 1 – 2048 DHCP pps) can also be configured. If exceeded, the port is put into err-disabled state.)

    - ip dhcp snooping trust (ports connected to authorized DHCP servers)

    - ip dhcp relay information trust-all

    - ip dhcp relay information trusted

    - no ip dhcp snooping information option

ARP Poisoning and MITM Attacks

- Attack

  - Bettercap in Kali (ARP Poisoning)

  - Driftnet in Kali (MitM Eavesdropping Attack)

- Defence

  - Dynamic ARP Inspection (DAI)

    - ip arp inspection trust

    - ip arp inspection limit rate rate (ARP rate-limit (default is 15 ARP pps). If exceeded, the port will transit to err-disabled state)

    - ip arp inspection validate ([src-mac] [dst-mac] [ip]) (validate contents of ARP reply matching layer 2 Ethernet addresses)

    - ARP ACL (Refer to pic)

IP and MAC Address Spoofing Attacks

- Attack

  - Set same ip or mac as the victim

- Defence

  - IP source guard

    - ip verify source (Against IP spoof)

    - ip verify source port-security (Against IP and MAC spoof)

    - ip source binding mac-address vlan vlan-id ip-address interface type number/number (Binds static ip and mac to a port)